CPE301 – SPRING 2023

# Design Assignment 2

Student Name: David Lenzin

Student #: 2001654470

Student Email: lenzin@unlv.nevada.edu

Primary Github address: https://github.com/dlenzin15/submissions

Directory: submissions/DA2

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB board was used. No other hardware was introduced. See schematic below for pin layout.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 2

Insert initial code here:

C Code:

/* * DA2_Task2_C.c * * Created: 3/6/2023 7:29:13 PM * Author : david */ #define F_CPU 16000000UL #include <avr/io.h> #include <util/delay.h> int main(void) { DDRC &= ~(1<<1); //Set PC5 to an input, which is connected to switch 1 PORTC ¦= (1<<1); //Pull-up resistor DDRB ¦= (1<<5); //Set PB5 as an output, which is connected to LED PORTB ¦= (1<<5); //Initially turn LED off while (1) { if (!(PINC & (1 << 1))) { //If switch is pressed PORTB &= ~(1 << 5); //Turn on LED _delay_ms(1750); //Delay for 1.75 seconds PORTB ¦= (1 << 5); //Turn off the LED } PORTB ¦= (1 << 5); //Keep the LED to off if button not pressed } }

Compilation:

Waveform:

Assembly Code:

; ; DA2_Task2_Assembly.asm ; ; Created: 3/6/2023 9:39:36 PM ; Author : david ; .include "m328pbdef.inc" ; Include the header file for the Atmega328PB board ldi r16, (1<<5) out DDRB, r16 ; Set PB5 as an output out PORTB, r16 ; Initially turn off the LED loop: sbis PINC, 1 ; check if switch at PC1 is pressed rjmp led_on ; jump to button_pressed if it is rjmp loop ; Loop forever led_on: ldi r16, ~(1<<5) ; Turn on the LED out PORTB, r16 rjmp delay ; Jump to delay function delay: ; Delay function to generate a delay of 1.75 seconds ldi r21, 143 ldi r22, 12 ldi r23, 66 L1: dec r23 brne L1 dec r22 brne L1 dec r21 brne L1 ldi r16, (1<<5) ; Turn off the LED out PORTB, r16 rjmp loop ; Return to loop

Compilation:

Waveform:

- DEVELOPED MODIFIED CODE OF TASK 3

Insert only the modified sections here

C Code:

/* * DA2_Task3_C.c * * Created: 3/6/2023 8:36:47 PM * Author : david */ #define F_CPU 16000000UL #include <avr/io.h> #include <util/delay.h> #include <avr/interrupt.h> ISR(INT0_vect) { PORTB &= ~(1<<5); //Turn LED on _delay_ms(3500); //Delay for 3.5 seconds PORTB ¦= (1<<5); //Turn LED off } int main(void) { PORTD ¦= (1<<2); //Activate Pull-up resistor for INT0 pin DDRB ¦= (1<<5); //Set PB5 as an output, which is connected to LED PORTB ¦= (1<<5); //Initially turn LED off EIMSK = (1 << INT0); //Enable interrupts on external pin INT0 EICRA = 0x03; //The rising edge of INT0 generate an interrupt request sei(); //Enable interrupts while (1); }

Compilation:

Waveform:

Assembly:

; ; DA2_Task3_Assembly.asm ; ; Created: 3/7/2023 9:04:30 PM ; Author : david ; .equ F_CPU = 16000000 .include "m328pbdef.inc" ;Include the header file .org 0x0000 jmp main .org 0x0002 ; External interrupt request 0 vector jmp INT0_ISR main: ldi r16, (1<<5) out DDRB, r16 ; Set PB5 as an output out PORTB, r16 ; Initially turn off the LED ldi r17, (1<<2) OUT PORTD, r17 ; Pull up for INTO pin ldi r18, (1<<INT0) ; load the bit mask for INT0 into r16 out EIMSK, r18 ; enable interrupts on external pin INT0 ldi r18, 0x03 ; load the value 0x03 into r16 sts EICRA, r18 ; set the rising edge of INT0 to generate an interrupt request sei ; enable interrupts globally loop: rjmp loop ; Infinite Loop INT0_ISR: ldi r16, ~(1<<5) ; Turn on the LED out PORTB, r16 ; Delay loops to generate a delay of 3.5 seconds ldi r21, 2 ldi r22, 29 ldi r23, 23 ldi r24, 133 L1: dec r24 brne L1 dec r23 brne L1 dec r22 brne L1 dec r21 brne L1 ldi r16, (1<<5) ; Turn off the LED out PORTB, r16 reti ; Return from Interrupt

Compilation:

Waveform:

- SCHEMATICS

Task 2:

Task 3:

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Task 2:

Task 3:

- VIDEO LINKS OF EACH DEMO

Link to Playlist: https://www.youtube.com/playlist?list=PLlHKEZIJ23uAW9Wtslh2ZcF6vdceCNBiI

Link to Task 2 C: https://youtu.be/OvwKFsnboKM

Link to Task 2 Assembly: https://youtu.be/ozF4z_ViWo8

Link to Task 3 C: https://youtu.be/cus2H9IzRUw

Link to Task 3 Assembly: https://youtu.be/A67nv6LcItk

- GITHUB LINK OF THIS DA

https://github.com/dlenzin15/submissions/tree/main/DA2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

David Lenzin