CPE301 – SPRING 2023

# Design Assignment 5

Student Name: David Lenzin

Student #: 2001654470

Student Email: lenzin@unlv.nevada.edu

Primary Github address: https://github.com/dlenzin15/submissions

Directory: submissions/DA5

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used:

- Atmega328PB board

- HC-SR04 ultrasonic sensor

- Smraza micro servo 9G

- Breadboard

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1

Insert initial code here

/* * DA5.c * * Created: 4/12/2023 10:31:35 AM * Author : david */ // Definitions #define F_CPU 16000000UL #define TRIGGER_PIN PB2 #define ECHO_PIN PB0 #define CONTROL_PIN PB1 #define BAUD 9600 #define MYUBRR F_CPU/16/BAUD-1 // Included Files #include <avr/io.h> #include <util/delay.h> #include <stdio.h> #include <avr/interrupt.h> // Function Declarations void UART_init(); void timer_init(); void UART_transmit_string(); void Wait(); uint32_t calculateDistance(); void UART_init(unsigned int ubrr) { //Set baud rate UBRR0H = (unsigned char)(ubrr>>8); UBRR0L = (unsigned char)ubrr; //Enable transmitter and receiver and receiver interrupt UCSR0B = (1<<RXEN0) ¦ (1<<TXEN0); //Set frame format: 8 bits data, 1 stop bit UCSR0C ¦= (1 << UCSZ00) ¦ (1 << UCSZ01); sei(); } void UART_transmit_string(char *data) { while ((*data != '\0')) { // Check if NULL char while (!(UCSR0A & (1 <<UDRE0))); // Wait for register to be UDR0 = *data; // Store data in the data register data++; } } void timer_init() { //Configure TIMER1 TCCR1A¦=(1<<COM1A1)¦(1<<COM1B1)¦(1<<WGM11); //NON Inverted PWM TCCR1B¦=(1<<WGM13)¦(1<<WGM12)¦(1<<CS11)¦(1<<CS10); //PRESCALER=64 MODE 14(FAST PWM) ICR1=4999; //fPWM=50Hz (Period = 20ms Standard). } //Simple Wait Function void Wait() { uint8_t i; for(i=0;i<2;i++) { _delay_loop_2(0); } } uint32_t calculateDistance() { PORTB &= (~(1<<TRIGGER_PIN)); _delay_us(2); // Pull trigger low before pulse /* Give 10 ms trigger pulse on trig. pin to HC-SR04 */ PORTB ¦= (1<<TRIGGER_PIN); _delay_ms(10); PORTB &= (~(1<<TRIGGER_PIN)); // Measure duration of pulse on echoPin unsigned long duration = 0; while (!(PINB & (1 << ECHO_PIN))); // Wait for echo to go high while ((PINB & (1 << ECHO_PIN))) duration++; // Measure pulse width uint32_t distance= (uint32_t)duration*0.034/2; return distance; } void main() { // Set data directions DDRB¦=(1<<CONTROL_PIN) ¦ (1<<TRIGGER_PIN); //PWM Pins as Out PORTD &= ~(1<<TRIGGER_PIN); // Pull down DDRB &= ~(1<<ECHO_PIN); // Set trigger pin to input timer_init(); UART_init(MYUBRR); int angle = 0; OCR1A = 97; // Initialize motor to 0 degrees while(1) { uint32_t distance = 0; char buffer[100]; sprintf(buffer, "Distance = %d cm\n", distance); UART_transmit_string(buffer); while (OCR1A < 535) { OCR1A += 5; // Increment every 2 degrees until we reach 180 degrees angle += 2; Wait(); distance = calculateDistance(); sprintf(buffer, "%d,%d.", angle, distance); UART_transmit_string(buffer); } OCR1A = 535; // 180 degrees angle = 180; while (OCR1A > 97) { OCR1A -= 5; // decrement every 2 degrees until we reach 0 degrees angle -= 2; Wait(); distance = calculateDistance(); sprintf(buffer, "%d,%d.", angle, distance); UART_transmit_string(buffer); } OCR1A = 97; // 0 degrees } }

- DEVELOPED MODIFIED CODE OF TASK 2

Insert only the modified sections here

import processing.serial.*; // imports library for serial communication import java.awt.event.KeyEvent; // imports library for reading the data from the serial port import java.io.IOException; Serial myPort; // defines Object Serial // Variables String angle=""; String distance=""; String data=""; String noObject; float pixsDistance; int iAngle, iDistance; int index1=0; int index2=0; PFont orcFont; void setup() { size (1200, 700); smooth(); myPort = new Serial(this,"COM3", 9600); // starts the serial communication myPort.bufferUntil('.'); // reads the data from the serial port up to the character '.'. So actually it reads this: angle,distance. } void draw() { fill(98,245,31); // simulating motion blur and slow fade of the moving line noStroke(); fill(0,4); rect(0, 0, width, height-height*0.065); fill(98,245,31); // green color // calls the functions for drawing the radar drawRadar(); drawLine(); drawObject(); drawText(); } void serialEvent (Serial myPort) { // starts reading data from the Serial Port // reads the data from the Serial Port up to the character '.' and puts it into the String variable "data". data = myPort.readStringUntil('.'); data = data.substring(0,data.length()-1); index1 = data.indexOf(","); // find the character ',' and puts it into the variable "index1" angle= data.substring(0, index1); // read the data from position "0" to position of the variable index1 or thats the value of the angle the Arduino Board sent into the Serial Port distance= data.substring(index1+1, data.length()); // read the data from position "index1" to the end of the data pr thats the value of the distance // converts the String variables into Integer iAngle = int(angle); iDistance = int(distance); } void drawRadar() { pushMatrix(); translate(width/2,height-height*0.074); // moves the starting coordinats to new location noFill(); strokeWeight(2); stroke(98,245,31); // draws the arc lines arc(0,0,(width-width*0.0625),(width-width*0.0625),PI,TWO_PI); arc(0,0,(width-width*0.27),(width-width*0.27),PI,TWO_PI); arc(0,0,(width-width*0.479),(width-width*0.479),PI,TWO_PI); arc(0,0,(width-width*0.687),(width-width*0.687),PI,TWO_PI); // draws the angle lines line(-width/2,0,width/2,0); line(0,0,(-width/2)*cos(radians(30)),(-width/2)*sin(radians(30))); line(0,0,(-width/2)*cos(radians(60)),(-width/2)*sin(radians(60))); line(0,0,(-width/2)*cos(radians(90)),(-width/2)*sin(radians(90))); line(0,0,(-width/2)*cos(radians(120)),(-width/2)*sin(radians(120))); line(0,0,(-width/2)*cos(radians(150)),(-width/2)*sin(radians(150))); line((-width/2)*cos(radians(30)),0,width/2,0); popMatrix(); } void drawObject() { pushMatrix(); translate(width/2,height-height*0.074); // moves the starting coordinats to new location strokeWeight(9); stroke(255,10,10); // red color pixsDistance = iDistance*((height-height*0.1666)*0.025); // covers the distance from the sensor from cm to pixels // limiting the range to 40 cms if(iDistance<40){ // draws the object according to the angle and the distance line(pixsDistance*cos(radians(iAngle)),-pixsDistance*sin(radians(iAngle)),(width-width*0.505)*cos(radians(iAngle)),-(width-width*0.505)*sin(radians(iAngle))); } popMatrix(); } void drawLine() { pushMatrix(); strokeWeight(9); stroke(30,250,60); translate(width/2,height-height*0.074); // moves the starting coordinats to new location line(0,0,(height-height*0.12)*cos(radians(iAngle)),-(height-height*0.12)*sin(radians(iAngle))); // draws the line according to the angle popMatrix(); } void drawText() { // draws the texts on the screen pushMatrix(); if(iDistance>40) { noObject = "Out of Range"; } else { noObject = "In Range"; } fill(0,0,0); noStroke(); rect(0, height-height*0.0648, width, height); fill(98,245,31); textSize(25); text("10cm",width-width*0.3854,height-height*0.0833); text("20cm",width-width*0.281,height-height*0.0833); text("30cm",width-width*0.177,height-height*0.0833); text("40cm",width-width*0.0729,height-height*0.0833); textSize(25); fill(98,245,60); translate((width-width*0.4994)+width/2*cos(radians(30)),(height-height*0.0907)-width/2*sin(radians(30))); rotate(-radians(-60)); text("30°",0,0); resetMatrix(); translate((width-width*0.503)+width/2*cos(radians(60)),(height-height*0.0888)-width/2*sin(radians(60))); rotate(-radians(-30)); text("60°",0,0); resetMatrix(); translate((width-width*0.507)+width/2*cos(radians(90)),(height-height*0.0833)-width/2*sin(radians(90))); rotate(radians(0)); text("90°",0,0); resetMatrix(); translate(width-width*0.513+width/2*cos(radians(120)),(height-height*0.07129)-width/2*sin(radians(120))); rotate(radians(-30)); text("120°",0,0); resetMatrix(); translate((width-width*0.5104)+width/2*cos(radians(150)),(height-height*0.0574)-width/2*sin(radians(150))); rotate(radians(-60)); text("150°",0,0); popMatrix(); }

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- The only output on the computer is from the distance graph.

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Peripherals:

Atmega328PB Board:

- VIDEO LINKS OF EACH DEMO

Complete Demo: https://www.youtube.com/watch?v=E6sbZxdOmO8&ab_channel=DavidLenzin

- GITHUB LINK OF THIS DA

https://github.com/dlenzin15/submissions/tree/main/DA5

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

David Lenzin